# Smart sensing technologies for Precision Crop Load Management in Australian Summerfruit Orchards

Smart sensing technologies are currently being evaluated (Project SF23000 - Precision Summerfruit Orchards) in measuring fruit and tree parameters. The primary objective of this assessment is to determine technologies that enhance outturn, production and labour efficiency.

The level of uniformity in tree size, fruit number, fruit diameter and fruit colour is being investigated within blocks of commercial Summerfruit crops (peach, nectarine, apricot and plum) in key growing regions (Sunraysia, Murray Valley, Goulburn Valley) of Australia. Pre-harvest and harvest scans of orchards using precision sensing technologies, enable real-time data collection on production uniformity, canopy characteristics, and provide agronomic insights for improved orchard management.

A survey was conducted on state-of-the-art commercially available smart technologies that use contact, proximal and remote approaches to measure canopy metrics and fruit and flower parameters.

The sensing technology being evaluated in project SF23000 includes mobile ground-based Cartographer (Green Atlas), drone (Outfield Technologies Orchard Management Platform) and smartphone APPs (Aerobotics, Hectre, Croptracker, DataLogger paired with Bluetooth digital calipers).

Technology comparisons and assessments

The following tables show sensing technology for fruit and tree parameters in relation to detection and data capture for flowers, fruit number, fruit size, fruit colour, tree size and yield forecasting. The tables have been grouped into three categories: handheld, proximal and remote sensing. Each table provides a list products and their sensor type, what they measure, sample size, and an overall rating – a precision agriculture score.

Technologies were evaluated according to:

- Data type captured

- Sample size

- Accuracy and resolution

- Cost

- Skill requirements

- Data integration

- Compatibility with existing farm management decision support software

- Tech support

Table 1. Handheld sensors that make direct physical contact with fruit or tree.

| Sensor | Scale | Sample size | Parameter | Precision Agriculture score (1–10) | Product |
| --- | --- | --- | --- | --- | --- |
| Bluetooth calliper | Fruit, Tree | Small | Fruit size, Trunk size | 3 | iGaging OriginCal Syntek Maxigear Mitutoyo U-Wave |
| Bluetooth colourimeter | Fruit | Small | Fruit colour | 5 | Rubens Technologies |
| Bluetooth spectrometer | Fruit | Small | Fruit maturity Fruit sweetness | 5 | Rubens Technologies |

Table 2. Proximal sensors that sense fruit or tree information in close range.

| Sensor | Scale | Sample size | Parameter | Precision Agriculture score (1–10) |  | Product |
| --- | --- | --- | --- | --- | --- | --- |
| Handheld |  |  |  |  |  |  |
| Smartphone | Fruit, Tree | Medium | Fruit number, Fruit size, Fruit colour, Tree size, Yield forecast | 5 – 7 | Aerobotics Croptracker Hectre Agerpoint Petiole ProTM FruitScout SoYield PixFruit |  |
| Terrestrial (e.g., ATV, tractor) |  |  |  |  |  |  |
| Camera, LiDAR | Blossom, Fruit, Tree | High | Blossom load, Fruit number, Fruit size, Fruit colour, Tree size | 6 – 10 | Green Atlas Cartographer Revolute Systems RevScout Fruitometry Intelligent Fruit Vision Aurea TreeScout Vivid machines Orion FruitMaps Bitwise GreenView FruitCast |  |

Table 3. Remote sensors that sense fruit or tree information at a distance.

| Sensor | Scale | Sample size | Parameter | Precision Agriculture score (1–10) |  | Product |
| --- | --- | --- | --- | --- | --- | --- |
| Drone, aircraft |  |  |  |  |  |  |
| Camera | Fruit, Tree | High | Fruit number, Fruit size, Tree size | 5 – 8 | Outfield Technologies Sentera Aerobotics |  |
| Satellite |  |  |  |  |  |  |
| Camera | Tree | High | Tree size | 5 | Ceres Imaging |  |
| Multispectral camera | Field | High | Yield forecast | 3 | Australian Tree Crop Map |  |

Image 1: Crop tracker smartphone APP for fruit count and fruit size

Image 2: Rubens Sensor and smartphone APP for measuring fruit sweetness, firmness and maturity

Image 3: Aerobotics smartphone APP for fruit size

Image 4: Outfield Technologies platform utilising drone for aerial sensing of tree size and fruit number

Image 5: Green Atlas Cartographer using sensors to scan blossom load

Image 6: Digital Bluetooth callipers with DataLogger smartphone APP for fruit size

Further information

Mark O’Connell, Agriculture Victoria mark.oconnell@agriculture.vic.gov.au

Acknowledgments

This project Precision summerfruit orchards (SF23000) has been funded by Hort Innovation, using the Summerfruit research and development levy, contributions from the Australian Government and co-investment from Agriculture Victoria. Hort Innovation is the grower-owned, not-for-profit research and development corporation for Australian horticulture.

These publications may be of assistance to you but the State of Victoria and its officers do not guarantee that these publications are without flaw of any kind or is wholly appropriate for your particular purposes and therefore disclaims all liability for any error, loss or other consequence which may arise from you relying on any information in these publications.